1.	INTRODUCTION	1

2. DEFINITIONS	3

3. CALCULATIONS OF PASSENGER LIFT	5

3.1. WIRE ROPE CALCULATION	5

3.2. SHEAVES CALCULATION	9

3.3. SLIDING CONTROL	13

3.4. CALCULATION OF MOTOR POWER	15

3.5. LIFT CAR DESIGN	16

3.5.1. DIMENSIONS OF LIFT CAR	16

3.5.2. DESIGN OF UPPER BEAM	16

3.5.3. DESIGN OF BOTTOM BEAM	19

3.5.4. DESIGN OF VERTICAL BEAM	19

3.6. GUIDEWAYS CALCULATION	22

3.6.1. LIFT CAR GUIDEWAYS CALCULATION	22

3.6.1.1. BUCKLING CONTROL GUIDEWAYS	23

2.6.1.2. STRESS CONTROL	24

3.6.1.3. DEFLECTION CONTROL	24

3.6.2. COUNTER WEIGHT GUIDEWAYS	25

3.6.6.1. STRESS CALCULATION	25

3.6.3.2. FASTENERS CONTROL	26

3.7. BUFFERS DESIGN	27

3.8. SHAFT DESIGN	29

4. CONCLUSIONS	33

# INTRODUCTION

Since the time man has occupied more than 1 floor of a building. He has given some consideration to vertical movement. The earliest terms were, ladders, stairways, animal powered hoist and manually driven windlasses. Ancient Roman ruins show signs of shaftways where some guided movable platform type of hoist was installed. Guides and vertical rails are a characteristic of every modern elevator. In Tibet people are up mountains in baskets drawn up by pulley and rope and driven by a windlass and manpower [1].

In modern elevators, security devices are wed to prevent it from falling in the event the lifting means or linkage fails. Elevator safety devices does not exist unit 1853 when Elista Groves invented the elevator safety devices. In today’s elevators both mechanical and electronically safety devices is wed [1].

A lift consists of mainly, lift car, guides, hosting, ropes driving elements, counter weight and a motor. Guides are placed into a well and lift car go up or down by means of wire rope. Counter weight is used to reduce motor power and buffer is placed base of well to prevent car against rapid strike. In addition some electrical or electronical elements are used for controlling movement, confortibility or safety.

Figure 1.1 Gearless elevator installation (Courtesy Otis Elevator).

# 2. DEFINITIONS

Before calculations, some terms related with passenger lift must be defined.

LIFT: Lift is a conveying machine, which carry both bad and people vertically or smaller than 15° to vertical direction, between predetermined points.

PASSENGER: Passenger is any person carried by lift.

LIFT CAR: Lift car is the part of lift, which carries load or passenger.

RATED LOAD: Guaranteed load value, which can be carried in lift by designer.

RATED SPEED: Guaranteed speed value of lift car by designer

WELL: well is a space which contains lift car, counter weight and guideways. The space is limited with base, ceiling and sidewall.

COUNTER WEIGHT: It’s a mass, which is used to reduce motor power.

GUIDEWAYS: Guideway is a part, which guide the lift car and counter weight.

PIT: pit is a space below the point that the lift car can go.

AVAILABLE AREA OF THE CAR: The area where Passenger use.

SPEED REGULATOR: It is a mechanism, which is used to brake lift car when car speed is higher than a predetermined value.

MACHINE ROOM: The room, which contains lift motor and control equipment.

BUFFER: Buffer is spring or hydraulic element, which prevent car against rapid strike to the pit base.

Not: The definitions translated into English from (TS 10922).

# 3. CALCULATIONS OF PASSENGER LIFT

In this project we will design a lift to carry 6 people along 15 floors.

Some properties, of the lift

Rated speed= 1,2 m/s

Lift car weight= 600 kg

Rated load= 6x80= 480 kg

Building Height= 15x3=45 m

Acceleration of the lift car= 0,4 m/s2

## 3.1. WIRE ROPE CALCULATION

The highest dynamic load on wire rope

S=Gcar+Q+Grg+ag

Where Gcar   = Lift car weight

Q	         = Rated Load

Gr	         = Mass of wire rope

g	         = Gravitation effect

a	         = Acceleration

Mass of wire rope is not known, so we will chose 6x15 FC regular lay with 8 mm diameter from Table 17-12 and Table 17-11

mass = 0,39 d2 = 0,39(8)2 = 24,96 kg/100 m =0,2496 kg/m

We will use 4 wire rope to carry lift (n=4)

Gr =(mass). lr. n

lr = length of 1 wire rope

n = rope number

Gr =(0,2494)(50)(4)= 50 kg.S=(600+480+50)9,81+0,49,81=1176kg                                                    = 11537 N

In each rope

T=Snumber of rope=115374=2884 N   Tu=T.s                s= factor of safety

In table 17,13 minimum factor of safety is given for  υ=1,2 m/s      s≃9,2 but s=12 have been chosen for safety

Tu=2884 x 12  Tu=34611   N=34,611 kN

Tu = A.d1,95

Tu	= Nominal strength

A 	= Stress coefficient

D	= Rope size

From table 17-12 for 6 x 19 FC

A=0,64334,611 = 0,643 d1,95

d=7,72 mm             d=8 mm  is chosen from table 17.11

The amount of wear that occurs depends ob the pressure of the rope in sheave groove

P=2FdD     D=34d   F= Tu

F	= Tensile force on rope

D	= Sheave diameter

d	= Rope diameter

D=34d  F= Tu		6,2=23461134.82=31,81 MPa

For 6 x 19 FC Regular lay rope with cast steel sheaves maximum allowable pressure is 6,2 Mpa (Table 17-14). Since 31,81>6,2 we have to increase rope diameter.

6,2=23461134.d2	d2=234611346,2⇒

d=18,12 mm        d=19 mm (Nearest standard value from table 17.11)

If a change Gr (Rope weight) will change and F will change

Mass of rope = 0,39 d2 = 0,39 (19)2 =140,79 kg/100 m

Gr=mass.lr.nGr=1,4079.50.4Gr=282 kg

S=600+480+282.9,81+0,49,81

S=1418 kg S=14,18 x 9,81=13906 N

In each Rope

T=S4=139064=3456,5NF=T.S    F=3476,5 x 12 =41718 N   P=2F34d2=    6,2=24171834d2     d2=241718346,2   d=19,89 mm

d= 22 mm is chosen (Standard value in table 17.11)

mass has change once again

mass =0,39 d2=0,39222=188,76kg100m                                            = 1,8876 kg/m  Gr=1,8876.4.50=378 kgS=378+480+6009,81+0,49,81S=1517 kg          S=14886 NF=148864x 12=44658 NP=2F34d2 d2=24465834.6,2d=20,58 mm

d=22 mm  is chosen

## 3.2. SHEAVES CALCULATION

From table 17.12 minimum sheave diameter for 6 x 19 FC

D=34dD=34.22=248mm

Guide sheaves diameter must be less than sheave diameter.

A sheave with 400 mm diameter will be used as guide sheaves

According to Turkish Standards (TS-1812) pressure stress on the sheave must be less than 18 kgf/cm2 for υ=1,2 m/s and keyed groove.

σ=Gcar+Q+Grn.d.D.1sinα2    TS-1812σ=600+480+3781,024.2,2.74,8.1sin352σ=7,51 kgf/cm2

n	= Number of wire rope

d	= Rope diameter

D	= Sheave diameter

α	= groove angle

7,51 kgf/cm2 < 80 kgf/cm2

So the sheave can be used

α=35°

Table 17-12 WIRE ROPE DATA (All Sizes In Millimeters)

| Rope* | Mass Kg/a00m | Min. dia. Sheave | Size Range d | Strength Coef. a | Elast. Mod. E, GPa |
| --- | --- | --- | --- | --- | --- |
| 6 x 7 FC | 0.35d2 | 42d | 6,5-38 | 0,608 | 80,6 |
| 6 x 7 IWRC | 0,39d2 | 42d | 6,5-38 | 0,656 | 68,6 |
| 6 x 19 FC | 0,39d2 | 34d | 6,5-70 | 0,643 | 74,4 |
| 6 x 19 IWRC | 0,43d2 | 34d | 6,5-70 | 0,688 | 93,0 |
| 6 x 19 IWRC↑ | 0,43d2 | 34d | 6,5-70 | 0,798 | 93,0 |
| 6 x 37 FC↕ | 0,39d2 | 23d | 6,5-83 | 0,643 | 68,2 |
| 6 x 37 IWRC↑↕ | 0,43d2 | 23d | 6,5-103 | 0,688 | 86,8 |
| 6 x | 0,43d2 | 23d | 6,5-103 | 0,798 | 86,8 |
| 8 x 19 FC | 0,37d2 | 27d | 6,5-38 | 0,520 | 55,8 |
| 8 x 19 IWRC | 0,42d2 | 27d | 13-29 | 0,579 |  |
| 8 x 19 IWRC↑ | 0,42d2 | 27d | 13-29 | 0,668 |  |

*Material is improved plow steel except as noted.

↑Material is extra-improved plow steel.

↕Equation (17-12) may not be valid for sizes greater than 70mm.

Source: Computed from data in Wire Rope Users Manual, American Iron and Steel Institue (AISI), 1979.

Table 17-13 MINIMUM FACTORS OF SAFETY FOR WIRE ROPE*

| Track cables | 3,2 | Passenger elevators, m/s: |  |
| --- | --- | --- | --- |
| Guys | 3,5 | 0,25 1,50 4,00 6,00 7,50 | 7,60 9,20 11,25 11,80 11,90 |
| Mine shafts, m: |  |  |  |
| Up to 150 300-600 600-900 Over 900 | 8,0 7,0 6,0 5,0 |  |  |
|  |  | Freight elevators, m/s: |  |
| Hoisting | 5,0 | 0,25 1,50 4,00 6,00 7,50 | 6,65 8,20 10,00 10,50 10,55 |
| Haulage | 6,0 |  |  |
| Cranes and derricks | 6,0 |  |  |
| Electric hoists | 7,0 |  |  |
| Hand elevators | 5,0 |  |  |
| Private elevators | 7,5 | Powered dumbwaiters, m/s: |  |
| Hand dumbwaiter | 4,5 | 0,25 1,50 2,50 | 4,8 6,6 8,0 |
| Grain elevator | 7,5 |  |  |

*Use of these factors does not preclude a fatigue failure.

Source: Complied from a variety of sources, including ANSI A17,1-1978

Table 17-14 MAXIMUM ALLOWABLE BEARING PRESSURES OF ROPES ON SHEAVES (MPa)

| Material |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Rope | Wood | Cast iron↑ | Cast steel↕ | Chilled cast iron ζ | Manganese steel ¶ |
| Regular lay: |  |  |  |  |  |
| 6 x 7 | 1,03 | 2,07 | 3,79 | 4,48 | 10,1 |
| 6 x 19 | 1,70 | 3,30 | 6,20 | 7,58 | 16,5 |
| 6 x 37 | 2,07 | 4,03 | 7,41 | 9,13 | 20,7 |
| 8 x 19 | 2,41 | 4,69 | 8,68 | 10,7 | 24,1 |
| Long lay: |  |  |  |  |  |
| 6 X 7 | 1,14 | 2,41 | 4,13 | 4,93 | 11,4 |
| 6 X 19 | 1,89 | 3,79 | 6,89 | 8,34 | 18,9 |
| 6 X 37 | 2,27 | 4,54 | 8,13 | 9,99 | 22,7 |

On end grain of beech, hickory, or gum.

↑ For HB(min.) = 125.

↨ 30-40 carbon; HB(min.) =160.

ζ Use only with uniform surface hardness

¶ For high speeds with balanced sheaves having ground surfaces.

Source: Wire Rope Users Manual, American Iron and Steel Institue (AISI), 1979.*

Table 17-11 PREFERRED MILLIMETER SIZES OF WIRE ROPE

6.5, 8, 9.5, 11.5, 13, 14.5, 16, 19, 22, 26, 29, 32, 35, 38, 42, 45, 48, 51, 54, 57, 61, 67, 70, 74, 77, 80, 83, 86, 90, 96, 103

Source: Wıre Rope Users Manual, American Iron and Steel Institue (AISI), 1979.

## 3.3. SLIDING CONTROL

Driving sheave

Guide sheave

S1

S2

Fig. 3.1

Counter weight

S2’

S1’

Fig.3.2

There are two critical points

1) Loading car at the bottom moving upwards (See figure 3.1)

α=35°

eμgrooveβ≥S1S2dyn must be

β=170°=2,965rad

μgroove=μsinα2    μ=0,09

μgroove=0,09sin352=0,3

Gcount=Gcar+Q2=600+4802=840kg

S1S2dyn=g+ag−a.Q+Gcar+GropeGcount            =9,81+0,49,81−0,4.480+600+378840=1,88

eμgrooveβ=e0,32,965=2,43

β=170°

2,43>1,87 no sliding

2) Empty car upper moving downward (See figure 3.2)

eμgrooveβ≥S2'S1'dyn  must be

S2'S1'=g+ag−aGcountGcar        =9,81+0,49,81-0,4840+378600        =2,2

eμgrooveβ=2,43

2,43≥2,2    no sliding

## 3.4. CALCULATION OF MOTOR POWER

N=1/ηP.ν102

P=F1−F                      	P	= the greatest rotational force

η	= system efficiency

γ	= velocity

F1=1,02Gr+Gcar+Q=1,02378+600+480=1487kgfF2=1,02Gcount=1,02840=857kgf

η=0,35   (From machine catalogue)

P = F1-F2 = 1487 – 857 = 630 kgf

N=10,35630x1,2102N=21,17kW≃22kW

## 3.5. LIFT CAR DESIGN

### 3.5.1. DIMENSIONS OF LIFT CAR

According to Turkish standards (TS 10922) max available car area for 450 kg rated load is 1,30 m2. Our rated load 480 kg so our area may be a bit higher and min. area for 6 people 1,17 m2. According to these values, we can use a lift car, like.

120 cm

110

Fig. 3.4

### 3.5.2. DESIGN OF UPPER BEAM

According to Turkish standards (TS 1812) upper beam bending stress must be lower than 900 kgf/cm2.

NPU 100 will be used for upper beam

σe = me / (n.w)     (TS 1812)

n	= 2 (Number of beam)

w	= 41,2 cm2 for NPU 100 (Section modules)

me = (1,02. Gt . L) / 4

Gt	=The greatest static load when car is top point

L	= 116 cm (Upper beam length)

Gt = [ (1,02) (1080) (116) ] / 4 =31978 kgf.cm

σe =31978 / (2 x 41,2) = 387 kgf/cm2

387 kgf cm2 ≤ 900 kgf/cm2 		so NPU 100 is appropriate

+

–

me

V

M

GT / 2

GT / 2

Q+Gt = 1080 kg

According to Turkish Standards (TS 1812) eL<11000 must be for upper beam of lift car.

e=1,02.Gt.L348x Ix

e	= Deflection of beam

Gt	= 1080 kg (Calculated before)

L	= 116 cm (Upper beam length)

For NPU 100 (St 37)

E	= 2,1 106 kgf/cm2

Ix	= 206 cm2

L	= 116 cm

e=1,021040116348.2,1.106206

e = 0,078

0,078116<0,0010,0006<0,001

So NPU 1000 can be used for upper beam material.

e

Gt / 2

Gt / 2

### 3.5.3. DESIGN OF BOTTOM BEAM

Strike of lift car to buffers cause stress on bottom beam.

According to Turkish standards (TS-1812) stress of bottom beam must be less than 1800 kgf / cm2

Bottom beam material is NPU 100 steel

σ=LGcar+Q+Gr2.W.nσ=116.600+480+3782.41.2.2σ=1026 kgf/cm21026<1800

NPU 100 can be used as upper beam material.

### 3.5.4. DESIGN OF VERTICAL BEAM

According to Turkish Standards (TS1812) total stress must be less than 1000kgf/cm2 and net cross-sectional area must be used in calculations.

L profile steels is used for vertical beams. We use L 50x50x5

Net cross-sectional area

F = Fn – a

Fn	= Cross-sectional area of beam

a	= Area of hole of fasteners

z	= Holes in 1 beam

d	= Hole diameter

t	= Material thickness

For L 50x50x5

Fn = 4,8 cm2

t = 0,5 cm

a=z.d.tz=41,22,4F=4,8−2,4=2,4cm2

In vertical beam both bending and normal stress occurs.

σtotal=M.H2.n.h.w+Qn.F

for passenger lift

M=Q.L8    (TS 1812)

H	= Free beam length

h	= Distance between car guide rollers

n = 2

h = 300 cm

H = 260 cm

L = 116 cm

For L 50x50x5 	W = 3,05cm3

M=1,0248011280=6732kgf.cm

σtotal=6732.2601.2.300.305+4802.2.4        = 580 kgf/cm2

580<1000 so L 50x50x5 appropriate

In addition

According to Turkish Standards (TS 1812)

LR≤120 must be

L = 260 cm (Free beam length)

R = The smallest inertial radius.

R=4Ix/A=411/2,4=4,28cm

A = 2,4 net area	Ix = 11 cm4 (For L 50x50x5)

2604,28≤120

60,74 < 120 so, L 50x50x5 appropriate

And, 
According to Turkish Standards (TS1812)

Ix'=m xH31,77nE.h≤Ix   must be

Ix=11.10−8m4   (For L 50x50x5)

m=Q.L8=480.1,168=66 kgm=66x9,81=647,4N.mE=2,1.108kpaIx'=647x2,63477,22,1.108.3=2,86.10−8m4

2,86.10−8<11.10−8

So we can use L 50x50x5 as vertical beam

## 3.6. GUIDEWAYS CALCULATION

### 3.6.1. LIFT CAR GUIDEWAYS CALCULATION

First we will found out force on 1 guideways during braking

P=Pfn 		where

P	= Force on 1 guideways during braking

Pf	= Total force during breaking

N	= Number of guideways

n =2

P=Q+Gcar+Gguide+m.b  (TS 1812)

Gguide	= Weight of guideway

m=Q+Gcarg=480+6009,81=110 kgs2mb=v22l(TS-1812)

According to Turkish Standards (TS-1812) v must be taken 1.4 times rated speed

v = (1,4).(1,2) = 1,68 m/s

l	= During sudden breaking the length that Lift car has to stop

l	= (According to Turkish Standards (TS 1812) l is maximum 15mm for    V=1,2 m/s rated speed)

b=1,6822.0,015=94,08Ms2

We need weight of guides. In this design T125x82x16 will be used as guideways.

Gguide= gguide.l

gguide	= weight of 1m guide

l	= length of guideways

for T 125x82x16

gguide	= 17,96 kg/m

l	= 50m (Approximately)

Gguide = (17,96).(50)

= 898 kg.

Pf = 12327 kg

Pf = 12574 kgf

P=125742=6287kgf                  =61675N

#### 3.6.1.1. BUCKLING CONTROL GUIDEWAYS

Pcr=π2.E.IxL2

Pcr	= Critical load

E	= 2,1.106 kgf/cm2(Elastic modules of guideways)

I	= 151,1 cm4 (Moment of inertia for T 125x82x16)

L	= 300cm (Length between guiderollers)

Pcr=π2.2,1.106.151,13002=34762

According to Turkish Standards PcrP≥5 must be

PcrP=347626287=5,5 	5,5 > 5 T 125x82x16   is appropriate

#### 2.6.1.2. STRESS CONTROL

According to Turkish Standards (TS-1812) maximum stress on guideways must be 350 kgf/cm2

σ=PFσ=628722,89   =274,6 kgf/cm2

σ	 = Stress on Guideways

F	 = Area of guideways

F	 = 22,89 cm2 (For T 125x82x16)

350 > 274,6    so t125x82x16 is appropriate

#### 3.6.1.3. DEFLECTION CONTROL

According to Turkish Standards maximum deflection must be 5 cm.

e=PL348.E.Ix (TS-1812)

P	= Force due to strike

L	= Length between guiderollers

P=1,02Q+Gcar2=550,8kgfL=300cme=550,3300348.2,1.106.151,1=0,97cm

0,97 < 5 so, T 125x82x16 is appropriate for lift car guideways

### 3.6.2. COUNTER WEIGHT GUIDEWAYS

Calculations are not necessary for counter weight, because there is no brake in counter weight guideways.

T 50x50x5 guideway can be used in counter weight guideways.

In order to connect 2 guideway each other we will use a flange as shown following figure.

#### 3.6.6.1. STRESS CALCULATION

σ=PA=628718,75=335.3 Kgfcm2A=(1,5).(12,5)=18,75 cm2

Material which is used to connect guideways is steel 37.

σmax=3700 Kgf/cm2 (For steel 37)

σcm= σmax/k=3700 / 5 = 740 Kgf/cm2

k	= Factor of safety

According to standards (TS 863)	k = 5

740 > 375,3 	so, steel 37 can be used as connection material.

#### 3.6.3.2. FASTENERS CONTROL

Fasteners will be used to connect guide ways.

σ=PA

σ	= Stress on a fasteners

A	= Total area

P	= Load in guide ways diving breaking

A = n . D . t

n	= 8 (Number of fasteners)

D	= Diameter of fasteners

We will use M16 fasteners

D=16 mm for M16 fasteners

A=8.1,6.1,5=19,2σ=628719,2=327,44 kgfcm2

According to Turkish standards (TS – 1812) stress must be less than

500 kgf/cm2

327,44 < 500

So, M16 fasteners can be used.

## 3.7. BUFFERS DESIGN

Spring buffers will be used According to Turkish standards (TS – 1812) buffers spring must not be closed under 2 (Gcar + Q) load. But under 3  (Gcar + Q) load springs must be closed.

BS 5216 grade 3 will be used spring material.

d = 12,5 	(Spring wire diameter)

D = 75 	(Mean spring diameter)

Number of coils = 12

N = 10 	(Number of active coils)

Closed spring length = (12). (12,5) = 150 mm

Stroke = 0,135 m = 135 mm 	(Stroke must be 0,135 for 1,2 m/s velocity according to TS – 1812)

k=d4G8D3N

k 	= spring constant

G	=79,3 GPa

F=k.xx=Fk=8D3Nd4G

4 buffer will be used parallel and each buffer takes equal loads. So, force will be F/4.

x=8FD3Nd4G

I. case

When  F = 2 (Gcar + Q) spring must be open

F = 2(600 + 480) = 1080 kg. = 21190 N.

x=8FD3N4d4G=21190.87510412,54.79,3.106=0,092 m

0,092 < 0,135

open so the spring is appropriate

II. case

When  F = 3(Gcar + Q) spring must be closed

F = 3600 + 480) = 3240 kg. = 31784 N.

x=8FD3N4d4G=(8)(31784)(75)3(10)4(12,5)4(79,3)(106)=0,1385 m

0,1385 > 0,135

Spring closed, so the spring can be used as buffer springs.

Now, whether spring can hold the load or not it must be checked.

Sut=Adm		For BS 5216a grade 3 (From table 10.2)

A	= 1980 MPa

m	= 0,179

Sut=198012,50,179=1260 MPa

Sy = 0,75 Sut = 945 MPa

τ=Ks8FDπd3τ=1,838(7946)(75)π(12,5)3=842 MPa

ks=1+0,5c=1+0,56=1,083c=Dd=7512,5=6F=317844=7946 N.

Sy > τ

So, BS 5216 grade 3 can be used as buffer spring.

## 3.8. SHAFT DESIGN

According to Turkish standards shaft of motor must be checked. Whether it can carry lift car load or not.

Shaft material St 50 CD 	Sy = 510 MPa ,  Sut = 650 MPa

Grope+ Gcar+ Q = 378+ 600+ 480

= 1458kg = 14303 N

Gcount=840 kg = 8240 N

A

FA

FB

B

P = 840 + 1458 = 2298 kg = 22543 N

D=22542

0,15m

0,15m

Mmax

+

–

V

M

FA

FB

T

FA=FB=225432=11271,5Mmax=112710,15=1691 Nm

Torque

T=D214303−8240=0,748214303+8240=2267 Nm

Based on static load only

d=32nπ sym2+T213	n = 5 (Factor of safety)

d=325π sy16,912+2267213=6,56 cm

According to TS1812 shaft diameter must be calculated based on static load only but here it will calculated based on fatique load for safety.

150mm

150mm

Key Groove

ka= 0,74kb= 1,18970−0,097=0,66

kc=0,753  	(Reliability 0,999)

kd=1ke=1kf=12,78=0,36

kf=1+qkf−1

kf=3  (For shaft subjected to both bending and torsion)

q =0,89kf=1+0,893−1=2,78

Se'=0,740,660,7530,36.510=67,52

d=32nπTSy2+mSe21213=10,9 cm

If we do one more interation

ka= 0,74kb= 1,189110−0,097=0,75

kc=0,753

kd=1ke=0,36

Se'=0,740,750,7530,36.510=76,72

d=32.5π22675102+169176,7221213

d=10,5

d≥110 mm must be at critical point.

# 4. CONCLUSIONS