Bu makalenin yayýnlandýðý kaynak :

ÝLKEN, Z., GÜNERHAN, H.,“An Investigation About the Relations Between the Results of Heat Conduction Problems With and Without Phase Change”, Int. Comm. in Heat and Mass Transfer, Volume 23, Number 6, ISSN 0735-1933, pp. 899-905, October 1996, Pergamon, USA.

AN INVESTIGATION ABOUT THE RELATIONS BETWEEN THE RESULTS OF HEAT CONDUCTION PROBLEMS WITH AND WITHOUT PHASE CHANGE

Zafer ILKEN

Dokuz Eylul University,Faculty of Engineering,Mechanical Engineering Dept.

35100-Bornova/Izmir/TURKEY

Huseyin GUNERHAN

Ege University,Solar Energy Institute

35100-Bornova/Izmir/TURKEY

ABSTRACT

There are physical relations between the velocities of the isotherms for two heat conduction problems having the same initial and boundary conditions, but one of them has phase change while the other one has not. The main idea of this study is to find these relations mathematically, which stand on a physical base, depending on the physical properties of the phases and the common parameters of these two different problems. If such kind of relations are known, then it will be possible to find the position of solid-liquid interface by using the analytical solutions and the relation itself, not solving the phase change problem. This idea is applied to heat conduction problem in semi infinite media and one dimensional phase change problem is solved for three different kind of boundary conditions via the Enthalpy Method. The results are correlated with the analytical solutions of the problem having same geometry and conditions but without phase change and obtained some relations having correlation coefficients changing between 0.91 and 0.98. This result shows that the regression analysis made is statistically meaningful.

Introduction

It is encountered with the solid-liquid phase change in many applications such as metallurgical processes, ice production, freezing of foods, latent heat energy storage etc. Therefore it composes an important field of heat transfer and finds a great interest by many researchers.

Phase change problems have nonlinear characteristics and therefore have only limited number of analytical or semi analytical solutions [1,2].Therefore many of them can only be solved by numerical techniques. The developments in computer technology accelerated the improvements in numerical analysis and this one is reflected to the solution methods used in phase change problems. Also new techniques are developed to obtain more precise results [3,4]. But many of the studies presented in literature belong to specific problems and/or ones having specific conditions. The studies presenting new methods or improvements of old methods can also be used in the specific type of problems. The generalized solutions are absent and therefore a problem encountered in any application should be solved again, unfortunately.

The idea of this study asserts that there is a relation between the results of a heat conduction problem with and without phase change, having same initial and boundary conditions. To obtain such a relation, the solidification problem in half space is solved for three different kind of boundary conditions; namely, constant temperature, constant heat flux and convective type of boundary conditions, and the change of position of solid-liquid interface is obtained. Then the same problem is solved as a pure conduction problem by assuming that the medium does not change phase at its phase change temperature, and the change of position of the isotherm corresponding to the phase change one is obtained. Then  multiple regression analysis are made including the problem parameters assuming that there should be a relation between these positions. As the result, some correlations are obtained and is shown that it is possible to obtain approximate solutions for interface position of a phase change problem by using these correlations and the analytical results of related pure heat conduction problem.

Interpretation of Mathematical Description and Analysis

Let us consider the solidification problem in a semi infinite region which has a uniform temperature of  Ti  ,initially  ( Ti > Tm  ). If solidification occurs under the third kind of boundary condition and the temperature of the ambient is T¥ ,( T¥ < Tm ),then the position of solid-liquid interface can be functionally described by means of the parameters included in the mathematical formulation :

,t > t1 	(1)

where t1  is the time at which the solidification begins (the surface temperature of the medium becomes equal to the phase change value).

If the same problem is handled as a pure heat conduction problem without phase change (In other words, if it is assumed that the medium does not change phase at Tm  or at lower temperatures than Tm ), then the position of the Tm  isotherm can be functionally described as,

, t > t1	(2)

The above relations can also be written in non-dimensional forms:

,t > t1	(3)

,t > t1 	(4)

The comparison of the functional relations (3) and (4) shows that there should be a strong relation between the isotherm motions of the heat conduction problems in half space with and without phase change. In fact, this relation depends on directly the physical properties of the material, the parameters including the physical properties and driving force of phase change (temperature difference).

In this study, solidification problem in half space is solved many times for different values of problem parameters belonging to each of three different kind of boundary conditions and a data set is formed, which consist of s values at definite times. The Enthalpy method of solution is chosen and the algorithm proposed by Voller  and  Cross [5] is used. Another data set is formed by using the analytical solutions [6] of  the heat conduction problem in half space under the same boundary conditions, which includes  values at redefined times. At the end, multiple regression analysis are made by using these data and the following correlations are obtained for three different kind of boundary conditions.

## For Constant Surface Temperature Boundary Condition

## S=0.47529.XFm0.96617.Fm−0.40984.Stes0.24804.(k+)−0.43552.(α+)−0.29406	(5)

For Constant Surface Heat Flux Boundary Condition

S=0.46443.XFm0.86579.Fm0.10112.q0.27602.(Ste)0.33702.(k+)−2.49963.(c+)5.48690	(6)

For Surface Convection Boundary Condition

S=0.74646.XFm0.96421.(Fm+)0.49360.Bi0.12823.Stel0.22447.(k+)−0.55944.(c+)1.24414	(7)

The range of applicability of the correlations and the regression coefficients of each correlation are given in Table-1. The limit values shown in this table are the ones used during program runnings.

### Table-1

| Correlation No. | 5 | 6 | 7 |
| --- | --- | --- | --- |
| Restrictions | 0.1≤Fm≤0.90.1≤Stes≤1.00.30≤k+≤0.931.18≤α+≤7.46 | −0.5≤Fm≤−0.1−10≤q≤−5000.01≤Ste≤1.00.30≤k+≤0.930.49≤c+≤0.88 | 0.1≤Fm+≤0.910≤Bi≤1000.1≤Stel≤1.00.30≤k+≤0.930.49≤c+≤0.88 |
| Regression Coeff. | 0.98 | 0.94 | 0.91 |

values included in the correlations (5-7) can be obtained by the use of analytical solutions given in Table-2.

Table-2

| Boundary Con | Analytical Solution [6] |
| --- | --- |
| First Kind | Fm=erf(x) (8) |
| Second Kind | Fm=qXFmx−1p−0.5exp−x2−erfcx (9) |
| Third Kind | Fm+=erfcx−exp(2xA+A2)erfc(x+A) , (10) |

t1 values for the correlations (6) and (7) can be obtained from (9) and (10) respectively, by setting  and x to zero in these equations.

Fig. 1.Comparison of the Correlation Curves with the Analytical Solution.

(A:Neumann Sol'n,fm=1.0;B1:fm=0.9;B2:fm=0.5;B3:fm=0.1)

The correlation (5) is plotted in Figure-1 for different values of superheat parameter . The well known Neumann Solution is also given in the same figure to make the comparison possible. The agreement between this analytical solution and the increasing trend of the slopes of correlation curves depending on increasing   also indicates the reasonability of the correlations to obtain approximate results.

Conclusion

The purpose of this study is to show the solutions of a heat conduction problem without phase change can be used to obtain the solutions of the phase change problem having the same geometry and conditions.The regression coefficients of the obtained correlations, which have the values between 0.91 and 0.98 confirm this idea. But all of  the computer programs are run by a personal computer having a microprocessor of 486 SX25. Therefore it is very restricted in choosing the number of grids used in numerical analysis and the number of data included in statistical analysis. These restrictions obviously affected the values of regression coefficients and  therefore the reliability of the correlations.But, since the main purpose of this study is to show the availability of reasonable correlations, finding more precise relations remained as a future work.

Nomenclature

Bi	Biot Number,=h.(unit length)/kl

Cl	Specific Heat of Liquid Phase [J/(kg°C)]

Cs	Specific Heat of Solid Phase [J/(kg°C)]

C+	=Cs/Cl

h	Convection Coefficient [W/(m²°C)]

kl	Thermal Conductivity of Liquid Phase [W/(m°C)]

ks	Thermal Conductivity of Solid Phase [W/(m°C)]

k+	=kl/ks

L	Latent Heat of Fusion [J/kg]

q	Heat Flux [W/m²]

q*	=q.(unit length)/(Ti.kl)

S*	Interface Position from the Surface [m]

S	=S*/(unit length)

Ste	Stefan Number

Stel	Stefan Number,=Cl.(Tm-T¥)/L

Stes	Stefan Number,=Cs.(Tm-To)/L

Ste*	Stefan Number,=Cl.(Ti)/L

Ti	Initial Temperature of the Liquid Half Space [°C]

Tm	Melting Temperature [°C]

To	Surface Temperature [°C]

T¥	Ambient Temperature [°C]

t	Time [sec]

XTm	Position of the Tm Isoterm from the Surface [m]

Xfm	=XTm/(unit length)

al	Thermal Diffusivity of Liquid Phase [m²/sec]

as	Thermal Diffusivity of Solid Phase [m²/sec]

a+	=as/al

f	Dimensionless Temperature

fm	=(Tm-To)/(Ti-To)

fm+	=(Tm-Ti)/(T¥-Ti)

fm	=(Tm-Ti)/(Ti)

x	=Xfm/(2.t)

t	Dimensionless Time,=al.t/(unit length)²

References

1.	M. N. Ozisik,Heat Conduction,John Wiley & Sons,New York,(1980).

2.	Virgil J. Lunardini,Heat in Cold Climates,Van Nostrand Reinhold Company,New York,(1981).

3.	V. R. Voller,C. R. Swaminathan & B. G. Thomas,”Fixed Grid Techniques for Phase         Change Problems:A Review”,Int. J. Numer.Methods Engng., 30 (4),875-898,(1990).

4.	A. A. Samarskii,P. N. Vabishchevich, O. P. Iliev & A. G. Churbanov, ”Numerical         Simulation of  Convection/Diffusion Phase Change Problems-A Review”,Int. J. Heat         Mass Transfer,36 (17),4095-4106,(1993).

5.	V. R. Voller & M. Cross,”Accurate Solutions of Moving Boundary Problems Using         Enthalpy Method”,Int. J. Heat Mass Transfer,24,545-556,(1981).

6.	H. S. Carslaw  & J. C. Jaeger, Conduction  of  Heat  in  Solids, Oxford   University Press, Second Ed.,London,(1959).

Bu makalenin yayýnlandýðý kaynak :

ÝLKEN, Z., GÜNERHAN, H.,“An Investigation About the Relations Between the Results of Heat Conduction Problems With and Without Phase Change”, Int. Comm. in Heat and Mass Transfer, Volume 23, Number 6, ISSN 0735-1933, pp. 899-905, October 1996, Pergamon, USA.