1. INTRODUCTION

The truss has been employed in America from the historic old wooden bridges to the vehicles of air travel. The first trusses were utilized in wood for buildings. With the large scale smelting of iron the truss gained great popularity among structural engineers. Today, the truss is engineering structure often utilized in the design of bridges, buildings, towers and cranes. The introduction of steel as a construction material of high tensile strength as well as compressive strength, a material that could be easily and strongly fastened together, and a material that could be produced in sections of varying shape, different cross sectional areas, and a multitude of lengths was the answer to truss construction.

Briefly, trusses carry their loads by developing axial forces in the bars. They are usually more economical than beams for moderately large spans. Their depth is larger than that of beams for the same span and load. They are common in factory buildings and bridges.

2. GENERAL INFORMATIONS

Trusses are structures composed of slender straight bars whose members are in tension or compression only. Truss members, which are two-force members, are connected at their extremities only. A truss is arranged in the form of one or more triangles.

Trusses are commonly employed in bridges, towers, and roof structures. They use material very efficiently and are consequently economical for spanning distances up to several hundred feet. Trusses may be used to span distances from 30 ft to 1000 ft. Trusses are usually made of steel or timber, and rarely of reinforced concrete because of its large dead load and low strength in tension. The bars of steel trusses may be steel trusses may be single, double angles or channels, tubes, or even I-sections. Timber truss members have rectangular cross-sections. Since steel members are usually more slender than timber members, truss configurations with shorter compressive members are more suitable for steel structures.

When a distributed load is to be supported by truss, as in the case of a bridge truss, a floor system must be provided which, through the use of stringers and floor beams, transmits the load to the joint.

2.1 Design Principles of Trusses

A structure that is composed of a number members that lie plane end are pin connected at their ends to form a stable framework is called a planar truss. For such structures to be stable, there can be no relative movement between any two points in the framework.

Most truss structures are three-dimensional in nature. In tower structures, microwave dishes, complicated roof structures, and aerospace structures, such analyses are in are in order. However, in many cases, such as bridge structures and simple roof systems, the framework can be subdivided into planar trusses without seriously compromising the accuracy of the results.

Truss structures can be indeterminate in two different ways. If there are more reaction component than can be determinate by consideration of statics, the structure is statically indeterminate externally. However, if there are more member forces than can be determined from statics, the structure is statically indeterminate internally.

2.2Idealizations for Planar Trusses

In reality, those structures that are collectively called truss-type structures are not composed of pin-connected members. In fact, the assumption of pin-connected members is only one several assumptions that are customarily made in the analysis of trussed frameworks.

The normal assumptions made in the analysis of planar trusses are enumerated below along with a comparison of how each of these idealizations relates to an actual structure.

a) Members are connected at their ends with frictionless pins: Even if they were pinned, some friction would be present. However this assumption gives reasonably good results for most truss-type connections.

b) Loads and reactions are applied to truss at the joints only: This realistic in terms of the applied loading because joints can be placed at points where loads are to be applied. For instance, a bridge might be composed of two parallel planar trusses as shown in Fig.2. These trusses are connected by floorbeams that span between them and frame into each truss at a joint.

c) The centroidal axis of each member is straight and coincides with the line connecting to the joint centers at each end of the member: To avoid eccentricity at a joint, the centroidal axes of all members fraiming into a joint should  intersect at a point.

Fig.1 Typical truss bridge structure

An analysis based on the assumptions of an ideal truss will yield the primary bar forces. An actual truss, which will be variance with the assumptions of an ideal truss, will have the bar forces that differ from the primary bar forces. These differences are the secondary bar forces. In most cases, the secondary effects are negligible when compared with the primary effects; however, the analyst must be sensitive to fact that they may be important in certain situations.

2.3. Variations in Planar Truss Configuration

The simplest form of a stable planar truss is a triangular arrangement of three members as shown in Fig.3a. Here, relative joint movements are preculled except for those that result from the member elongations that accompany the material strains.

In contrast, the four-member arrangement of Fig. 3b. admits relative joint movements through a realignment of the members as shown by the dashed lines.

If it is required to enlarge the structure of Fig. 3s to included an additional joint d, as shown in Fig. 3c, then two new members must be added to the truss. The structure of Fig. 3d results from adding members ed and ea to establish joint e and members fb and fe include joint f.

Fig. 2 Arrangement of members for simple planar trusses. (a) Basic triangular arrangement. (b) Four member arrangement. (c) Enlargement of truss to include joint at d. (d) Further expansion.

A structure formed by this basic procedure is called a simple truss Of course, in forming a simple truss, the original triangular unit must connect joints that are not located along a straight line, and each added joint must not lie along a line passing through the two joints to which it is being connected.

Other stable truss configurations are possible that do not evolve from the basic procedure described above. For instance, a compound truss results when two or more simple trusses are joined together to form a stable framework.

2.4. Some Typical Trusses

Fig.3 Typical trusses

2.5. Truss Joint Identification and Member Force Representation

There are numerous ways to identify the truss joints and to represent symbolically a given bar force. Any notation scheme is acceptable if it effectively communicates the meaning and if the analyst is consistent in its use.

In the development given here, each truss joints and to represented by the letter F with a double subscript appended to designate the bar. For instance Fij represents the force in the member that connects joints i and j.

Fig.4 (a) Joint identification. (b) Bar notation.

If a free-body diagram cuts through a member then the force in that member must be shown to act on the cut face. Fig. 4b shows a section through member ij and the force Fij acting on the cut face of the member. In resolving a bar force into these components the horizontal component is designed by X and the vertical component by Y. Again, a double subscript is appended to represent the bat. Fig. 6b shows the bar force Fij along with its components Xij and Yij.

Because the bar force and member direction are concident, similar triangles exist between the member and its geometric component and the bar force and its components. These similar triangles are lightly shaded in Fig.6. From these similar Triangles, the following relationships exist between the bar and its components;

FijLij=Xijxij=Yijyij

(1.1)

From this, any element of the force triangle can be determined from any other. That is;

Fij=XijLxij=YijLyijXij=FijxLij=YijxyijYij=FijyLij=Xijyxij

(1.2)

2.6. Sign Convention and Force Representation

Along with a comprehensive and consistent notation, a sign convention must be adapted. In truss analysis, the sign convention that is almost universally employed designates a tensile force as a positive bar force and a comprehensive force as a negative bar force. This notation has a convenient physical interpretation in which a positive force is accompanied by a lengthening of the member.

Tension

Compression

Fig. 5 Bar force representation. (a) Action of joints on member.                      (b) Action of member joints.

First, it means that the correct direction was assumed, and second, it means that the member is elongated or in tension. On the other hand a negative force means that the assumed direction was incorrect and that the member is being shortened in compression.

Sometimes it is necessary to isolate a member and represent the forces acting on it. There are two ways in which it is convenient to represent such member forces. In the first, the forces that are exerted on the member by the joints are indicated, whereas in the second, the actions of the member on the joints are shown. These two approaches are shown in Fig.5.

2.7. Strategy for Planar Truss Analysis

The term truss analysis is used to describe the process whereby the bar forces in a truss are determined. The process requires repeated use of free-body diagrams from which individual bar forces are determined.

In some cases, a single joint is isolated as shown in Fig. 8a. Here, the free-body is acted on by a concurrent force system and moment equilibrium is automatically satisfied. Thus, for planar truss, two independent equilibrium equations are available, and only two unknown bar forces can be determined. The method of joints is a technique of truss analysis in which the bar forces are determined by the sequential isolation of joints the unknown forces at one joint are determined and become known bar forces at subsequent joints.

Fig.6 Typical free body diagrams. (a) Section1.1 (b) Section 2.2

In order cases, a larger portion of the structure is isolated by taking a section through the structure as shown in fig.8b. Here, all three equations of static equilibrium are available for the planar case and thus three bar forces can be determined.

In many cases, it is found that a combination of the method of joints and the method of sections is employed. One should determine the desired bar forces by applying the equations of equilibrium to the most convenient free-body diagrams and should not be committed to some particular method.

2.8 Statical Determinacy and Stability of Planar Trusses

The most fundamental from of a stable planar truss is the simple triangular arrangement of three members shown in Fig. 7a. In this case, there are six unknowns (three bar forces and three independent reaction components) and six independent equations or equilibrium (two equations at each joint), and thus the system is statically determinate. Of course the reactions could be determined first by applying the conditions of static equilibrium to the entire structure, and then bar forces could be determined by joint considerations. In either case, however there are only six independent equations of equilibrium.

If it is required to enlarge the truss to include an additional joint d, as shown in Fig. 10b, then tqo members must be added to the truss. They new furnishes two new equations of equilibrium for the determination of the two new bar forces.

Fig.7 Formation of statically determine planar truss. (a) Stable truss               (b) Enlargement of truss

If a reaction component were added without including a condition that would enable its determination, the structure would be statically indeterminate externally. Or if a member were added without increasing the number of joints, the structure would become statically indeterminate internally.

The structure can be tested for external determinacy and stability. That is, if r is taken as the least number of reaction components, then the following criteria hold:

ra < r ; structure is statically unstable externally

ra = r ; structure is statically determinate externally

ra > r ; structure is statically indeterminate externally

As pointed out earlier, the conditions for ra ≥r are necessary but not sufficient conditions for statical classification. The reactions must be properly arranged to ensure stability.

For internal classification, in addition to above definition for r, let m be the total number of bars and j the total number of joints. Then

2j = m + r 						(1.3)

is the relationship that must be satisfied if the truss is to be statically is to be statically determinate internally. Equation 1.3 can be rewritten as

m = 2j – r 						(1.4)

In this form, m is the number of members required to form an internally statically determinate truss that connects j joınts and has r reaction components required for external stability. If ma is the actual number of bar forces in the truss, then the following conditions prevail:

If ma<m, the truss is definitely unstable, but if ma ≥m it does not necessarily follow that the truss is stable.

Table (1.1) illustrates the statical classification of several planar truss structures. In each case, both external and internal classification are considered according to the criteria that have been developed. It should be recalled that r is equal to three (corresponding to the three equations of statistic equilibrium) plus the number of additional condition equations available for the determination of the reactions.

Table 1.1 Statical Classification of trusses

3.EXAMPLES

# Example problem 3.1:

Determine the forces in members U1L2 and U1U2 for the truss and loading shown. The structure is statically determinate both internally and externally, and the reactions are given.

## Example Problem 3.2

Determine the bar forces in members ad and bd for the tower shown. The structure is statically determinate.

## Example problem3.3

Determine the force in members EF and GI of the truss shown

Free-Body : Entire truss.  A free-body diagram of the entire truss is drawn; external forces acting on this free body consists of the applied loads and the reactions at B and J. We write the following equilibrium equations.

+↑MB=0:−28kips8ft−28kips24ft−16kips10ft+J32ft=0

J =+33kips    J =33 kips↑

+Fx=0:          Bx =-16 kips    Bx = 16 kips←

+↑Mj=0:28kips24ft−28kips8ft−16kips10ft+J32ft=0

By = +23 kips   By = 23kips↑

Force in Member EF. Section nn is passed through the truss so that it intersects member EF and only two additional members. After the intersected members have been removed, the left-hand portion of the truss is horizontal forces, we write

+↑Fy=0:

+23kips−28kips−EEF=0EEF=−5kips

The sense of FEF was chosen assuming member EF to be in tension; the negative sign obtained indicates that the member is in compression.

FEF =5 kips C

Force in Member GI. Section mm is passed through the truss so that it intersects member GI and only two additional members. After the intersected members have been removed, we choose the right-hand portion of the truss as a free body. Three unknown forces are again involved; to eliminate the two forces passing through point H, we write

+↑MH=0:33kips8ft−16kips10ft+FGI10ft=0         FGI = – 10 .4  kips  	FGI =  10.4 kips C

4. Complex Planar Trusses

There are planar truss configurations that are neither simple nor compound. For instance, consider the simple truss shown Fig.3.1.a. Beginning with the triangular unit abc, the truss is systematically assembled by adding two members to establish each new joint. A count of the members and joints verifies that Eq (1.4) is satisfied, and therefore the structure is statically determine. If member cd is removed and a member connecting joints e and f is added, shown in Fig.3.1.b, the structure remains statically determine; however, its form is no longer that of a simple truss. In addition, it is not a compound truss. Such a configuration is referred to as a complex truss. Complex trusses can be analyzed by a strategy involving a combination of the method of joints and the method of sections.

Fig.3.1 Formation of a complex planar truss. (a) Simple truss. (b) Complex truss.

5 STİFFNESS METHOD FOR TRUSSES

5.1 Development of stiffness Matrix

Development of the stiffness method for trusses involves the derivation of the truss element stiffness matrix, transformation of coordinates, and equilibrium of the joints. The solution of equilibrium equations gives displacements, which in turn lend to the member forces.

5.2 Truss Element Stiffness Matrix

N1,V1                         N2,V2

F = 0                                            N1= -N2 					                      (5.1)

V2 = ( N2L/EA) + V1							      (5.2)

V2-V1 = N2L/EA

From equation (5.2);

N2 = (EA/L)(V1/V2)					                              (5.3-a)

From equations (5.1) and (5.2)

N1 = (EA/L) (V1/V2)							      (5.3-b)

Expressing the above equations in matrix form, we find;

N1N2=EAL1−1−11V1V2

(5.4)

Force-displacement relation for a truss element;

P=kd

(5.5)

P ; Element force vector in local coordinates

k  ; Element displacement vector in local coordinates

d  ; Element stiffness matrix in local coordinates

k=EAL1−1−11

(5.6)

=k11k12k21k22

From equations (5.4) and (5.2),

If

V1=1 and V2=0    ⇒   N1=P1=k11

N2=P2=k21

L,     E,     A

L,     E,     A

V1=d1=1                                      V2=d2=0

N1=P1=k11                                                  N2=P2=k21    ⇒k11+k21=0

EAL−EA−L=0

If

V1=0 and V2=1;

N1=P1=k12

N2=P2=k22

L,     E,     A

L,     E,     A

V1=d1=0                V2=d2=1

N1=P1=k12                                                                    N2=P2=k22⇒ k12+k22

−EAL+EAL=0

P=P1P2    d=d1d2

N2=P2=k22  ⇒   k12+k22=0

d1     d2

P1 P2

Note;

k−1    ; Stiffness matrix

f	; Flexibility matrix

Flexibility element equation is;

d=fP

5.3 Transformation of Coordinates

In a truss the bars have different orientations. Thus, before we write the equilibrium of joints, the bar forces must be decomposed into their components in a global coordinate system. The global axes are chosen in the horizontal and vertical directions, directed to the right and upward.

D4 d2                 P4  P2

2             d3                              2               P3

1                                                                     1

d2                                                                     P2

d1                                P1

d1                                                                  P1

y

x

cos α = c

sin β = s

cos2 α + sin2 β = 1

The global x-axis is directed from the first to the second end of the member. The first and second ends can be chosen arbitrarily. The angle is measured from the global to the local x-axis in a counter clockwise sense.

If the joints coordinates are (x1,y1) and (x2,y2), then

c = cos α = (x2-x1)/L

s = sin α = (y2-y1)/L

(5.7)

and the length of the member is;

L = ((x2-x1)2+(y2-y1)2)1/2

(5.8)

The end displacements along the axis of the bar and forces can be expressed in terms of their global displacements and forces as.

d1=V1cos∞=d1cos∞d2=V1sin∞=d1sin∞                                d3=V2cos∞=d2cos∞d4=V2sin∞=d2sin∞

(5.9)

d1d2d3d4=cs0000cs.d1d2

(5.10a)

d=Td

(5.10.b)

d  ;   global

d  ;   local

d T   ;   transformation matrix

d=T−1d=TTd

(5.10c)

where

TT=c0s00c0sd=d1d2=c0s00c0sd1d2d3d4

in a similar manner;

P=P1P2P3P4cs00s0csP1P2=TP

(5.11)

P=TTPP1P2=c0s00c0sP

(5.12)

Consider equation (5.5);

P=kd

Equation (5.10c) and (5.12) into (5.5) ;

TTP=kTTdP=TkTTdP=kd

(5.13a)

Where the element stiffness matrix in global coordinates can be obtain as;

k4+4=T4+2=k4+2T4+2Tk=EAL1−1−11k=TkTT            			                 (5.13b)

Tk=EA/Lcs0000cs1−1−11           =EALcs−c−s−c−scs

T4∗4k4∗4T4∗4T=EA/Lcs−c−s−c−scscs0000cs

=EA/Lc2cs−c2−cscss2−cs−s2−c2−csc2cs−cs−s2css2

k=EA/Lc2cs−c2−cscss2−cs−s2−c2−csc2cs−cs−s2css2

(5.14)

5.4 Assembly of elements and Solution Procedure

- Number the elements, joints and degrees of freedom, that is, the possible horizontal and vertical displacement for each joint, in the global coordinate system. The horizontal degree of freedom is numbered before the vertical degree of freedom.

- Find the element force-displacement relationship in the global coordinates, equations (5.13a) and (5.14), and write the degrees of freedom in the element displacement vectors above the columns of the stiffness matrixes.

- Setup the equilibrium equations in terms of the structure stiffness matrix and external force or load vector. The rank of the structure stiffness matrix is equal to the number of degrees of freedom. The elements of the structure stiffness matrix are found by adding the terms of the element stiffness matrixes associated with that degree of freedom. The elements of the load vector consist of the joint loads in the degree of freedom associated with each term.

- Solve equilibrium equations to find the joint displacements.

- Determine the element displacement vectors in the global coordinates from the above joint displacements, by adding zeros for the fixed joints.

- Find the member-end forces by equations (5.5) and (5.12)

## EXAMPLE FOR THIS STUDY

Truss is loaded as shown, determine the force in members EF, FH, CE, and EH?

Fy=  0⇒ FAy=2t+2t+2t ⇒  FAy=6t

+↑MA=  0 ⇒ FB x 6.25  -  2t x  6  -  2t x  9  -  2tx  12   =0

FB=54t6.25⇒FB=8.64t

+↑MB=  0⇒ -2t x 6 - 2t x 9 - 2t x 12 + FAx x 6.25  =  0

FAX=54t6.25⇒FA=8.64t

FAY

FAX       	                  FCE

k

FEF

V FFH

FB

FY= 0 ⇒ FEF= 6t +↑MC=  0⇒ FB x 6.25 + FFH x 4 - FAY x 3 − FEF x 3  =  0

⇒  8.64t x 6.25 + FFH x 4 - 6t x 3 - 6t x 3 =0

⇒  FFH=-184  ⇒ FFH=−4.5t

+↑MF=  0⇒ FAX x 4 + FB x 2.25 - FAY x 6 − FCE x 4 =  0                         ⇒  8.64t x 4 +8.64 x 2.25 - 6t x 6 - FCE x 4 =0                                        ⇒  FCE=184  ⇒ FCE=4.5t

FAY  						2t

FEG

37°

FAX

FEH

FFH

FB

FY=  0⇒ FAY -2t -FEH x cos37 =0                      6t - 2t-FEH x cos37  =0                      FEH x cos 37 =  4t ⇒ FEH=5t